Assignment 5: Network Fundamentals and Building Networks

Section 1: Multiple Choice

1. What is the primary function of a router in a computer network?

a) Assigning IP addresses to devices

b) Providing wireless connectivity to devices

c) Forwarding data packets between networks

d) Managing user authentication and access control

ANS: c) Forwarding data packets between networks

2. What is the purpose of DHCP (Dynamic Host Configuration Protocol) in a computer network?

a) Assigning static IP addresses to devices

b) Resolving domain names to IP addresses

c) Managing network traffic and congestion

d) Dynamically assigning IP addresses to devices

ANS: d) Dynamically assigning IP addresses to devices

3. Which network device operates at Layer 2 (Data Link Layer) of the OSI model and forwards data packets based on MAC addresses?

a) Router

b) Switch

c) Hub

d) Repeater

ANS: b) switch

4. Which network topology connects all devices in a linear fashion, with each device connected to a central cable or backbone?

a) Star

b) Bus

c) Ring

d) Mesh

ANS: b) bus

Section 2: True or False

1) A VLAN (Virtual Local Area Network) allows network administrators to logically segment a single physical network into multiple virtual networks, each with its own broadcast domain. True

2) TCP (Transmission Control Protocol) is a connectionless protocol that provides reliable, ordered, and error-checked delivery of data packets over a network. False

3) A firewall is a hardware or software-based security system that monitors and controls incoming and outgoing network traffic based on predetermined security rules. True

Section 3: Short answers

1) Describe the steps involved in setting up a wireless network for a small office or home office (SOHO) environment.

ANS:

1. Choose the required hardware

- Wireless router (Wi-Fi router)

- Internet connection from ISP (fiber/DSL/cable)

- Modem (if not integrated with router)

- End devices (laptops, mobiles, printers)

2. Connect the hardware

- Connect ISP line → Modem

- Connect Modem → Router (WAN port)

- Power on modem and router

- Use LAN cable for initial setup (recommended)

3. Access router configuration page

- Open a web browser

- Enter router IP address (e.g. 192.168.1.1)

- Login using default username and password

4. Configure internet (WAN settings)

- Select connection type:

  - DHCP (most common)

- Save settings and verify internet access

5. Configure wireless settings

- Set SSID (Wi-Fi name)

- Select appropriate channel (auto or manual)

6. Secure the wireless network

- Enable WPA2 or WPA3 encryption

- Set a strong Wi-Fi password

- Disable WPS (optional but recommended)

- Change default router login password

7. Configure IP addressing

- Enable DHCP server on router

- Router will automatically assign IP addresses to devices

8. Connect devices

- Search for Wi-Fi network on devices

- Enter Wi-Fi password

- Verify connectivity

9. Test the network

- Check internet access

- Test file sharing or printing (if required)

- Ensure good signal coverage

10. Optimize and maintain

- Place router in central location

- Update router firmware

- Set firewall rules if required

Section 4: Practical

1) Demonstrate how to configure a router for Internet access using DHCP (Dynamic Host Configuration Protocol).

Step 1: Connect interfaces

- WAN interface (e.g., GigabitEthernet0/0) → ISP / Modem

- LAN interface (e.g., GigabitEthernet0/1) → Switch / PCs

Step 2: Access router CLI

Router> enable

Router# configure terminal

Router(config)# interface gigabitEthernet0/0

Router(config-if)# ip address dhcp

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# ip dhcp pool LAN_POOL

Router(config-dhcp)# network 192.168.1.0 255.255.255.0

Router(config-dhcp)# default-router 192.168.1.1

Router(config-dhcp)# dns-server 8.8.8.8

Router(config-dhcp)# exit

Section 5:

1) Discuss the importance of network documentation in the context of building and managing networks.

ANS:

1. Helps in network planning and design

- Provides a clear view of network topology

- Helps administrators understand how devices are connected

- Makes future network expansion and upgrades easier

2. Simplifies troubleshooting

- Allows quick identification of devices, IP addresses, and connections

- Reduces downtime by speeding up fault isolation

- Helps new technicians understand the network faster

3. Improves network management

- Keeps record of router, switch, and firewall configurations

- Helps in change management

- Ensures consistency in network operations

4. Enhances network security

- Documents security policies, firewall rules, and access controls

- Helps identify unauthorized devices or changes

- Supports security audits and compliance

5. Supports disaster recovery

- Helps in restoring the network quickly after failures

- Provides backup information for configurations

- Reduces recovery time during emergencies

6. Saves time and cost

- Reduces dependency on a single administrator

- Minimizes errors during configuration

- Lowers maintenance and operational costs

7. Knowledge transfer and training

- Helps new staff learn the network easily

- Ensures continuity when administrators change